THERMAL-HYDRAULICS AND LOCA SIMULATOR

Possible Student Exercises

Open Nuclear Engineering Teaching Suite

Undergraduate teaching guide

A structured set of 38 activities covering normal operation, heat transfer, reactor trip and decay heat, LOFA, LOHS, SBLOCA, LBLOCA, station blackout, residual heat removal, data analysis and operator decision-making.

Prepared for classroom, laboratory and guided self-study use

# Purpose and scope

The Thermal-Hydraulics and LOCA Simulator supports undergraduate exercises by coupling reactor power and decay heat with fuel, cladding and coolant temperatures; primary flow, pressure and inventory; steam-generator heat removal; emergency core cooling; auxiliary feedwater; pressure-control systems; and residual heat removal.

| Capability | Teaching use |
| --- | --- |
| Scenario presets | Normal operation, SBLOCA, LBLOCA, loss of flow, loss of heat sink and station blackout. |
| Protection and safety systems | Automatic or manual reactor trip, automatic or manual ECCS, auxiliary feedwater, PORV, spray and RHR. |
| Dynamic displays | Power, decay heat, fuel/cladding/coolant temperature, pressure and coolant inventory. |
| Manual controls | Rod insertion, reactivity trim, boron, pump flow, SG heat removal, break size and safety-system actuation. |
| Data logging | CSV output for calculation, plotting, chronology reconstruction and laboratory reporting. |

Important limitation

This is a simplified lumped-parameter classroom model. It is intended to teach qualitative system behaviour and cause-and-effect relationships. It must not be used for design, licensing, safety analysis, plant procedures or operator qualification.

# A. Basic thermal-hydraulics exercises

## 1. Establishing the normal operating condition

Learning objective: Recognize the normal steady-state relationships between heat generation, heat transfer and heat removal.

Activity: Run the simulator at nominal settings and allow the displayed quantities to stabilize.

Record or compare: Reactor power, decay heat, fuel temperature, cladding temperature, coolant temperature, primary pressure, coolant inventory and primary flow.

## 2. Effect of reactor power on temperatures

Learning objective: Relate reactor power to the thermal gradients across the fuel, cladding and coolant nodes.

Activity: Use the control rods or fine-reactivity trim to change power while keeping primary flow and steam-generator heat removal constant.

Record or compare: Plot fuel, cladding and coolant temperature against reactor power.

Questions for discussion

Which temperature responds first?

Which temperature shows the largest change?

Why does fuel temperature remain above cladding temperature?

Why does cladding temperature remain above coolant temperature?

## 3. Effect of primary coolant flow

Learning objective: Relate coolant flow to convective heat-transfer effectiveness.

Activity: Reduce reactor coolant pump flow in stages: 100%, 80%, 60%, 40% and 20%. Keep other controls as constant as practical.

Record or compare: Fuel, cladding and coolant temperatures at each flow setting.

Expected observation: Lower flow generally produces poorer heat removal and higher fuel and cladding temperatures.

## 4. Natural-circulation cooling

Learning objective: Explain how heat can continue to be transported after loss of forced circulation.

Activity: SCRAM the reactor, reduce primary pump flow to zero and retain steam-generator heat removal.

Questions for discussion

Does the indicated flow fall completely to zero?

Why can heat still be transferred without forced circulation?

Is natural circulation sufficient for full-power operation or mainly for shutdown cooling?

## 5. Effect of steam-generator heat removal

Learning objective: Connect secondary-side heat removal with primary-system temperature and pressure.

Activity: Hold reactor power and primary flow approximately constant while setting SG heat removal to 100%, 75%, 50%, 25% and 0%.

Record or compare: Coolant temperature, primary pressure, fuel temperature and cladding temperature.

# B. Reactor trip and decay-heat exercises

## 6. SCRAM and decay-heat response

Learning objective: Distinguish the rapid reduction of fission power from the slower removal of residual decay heat.

Activity: Run at full power, press SCRAM and observe the transient.

Record or compare: Fission power, decay heat, fuel temperature, cladding temperature and coolant temperature.

Key lesson: Reactor shutdown does not mean that heat generation immediately becomes zero.

## 7. Cooling after SCRAM with and without heat removal

Learning objective: Demonstrate that shutdown alone is insufficient; decay heat must continue to be removed.

Activity: Compare two cases: (1) SCRAM with SG heat removal maintained, and (2) SCRAM followed by loss of SG heat removal.

Record or compare: Peak coolant pressure, peak coolant temperature and peak cladding temperature.

## 8. Delayed operator response after trip

Learning objective: Evaluate the effect of operator response time on the severity of a post-trip heat-removal transient.

Activity: After SCRAM and loss of heat removal, delay auxiliary feedwater initiation by 10 s, 30 s and 60 s.

Record or compare: Peak coolant temperature, peak pressure, peak cladding temperature and time required for recovery.

# C. Loss-of-flow exercises

## 9. Complete loss-of-flow accident

Learning objective: Trace the thermal response following loss of forced primary circulation.

Activity: Use the LOFA preset and observe pump-flow reduction, reactor trip, temperature response, natural circulation and SG heat removal.

Questions for discussion

Why can cladding temperature rise even after the reactor trips?

Which is more important immediately after LOFA: reducing power or restoring cooling?

How does coolant inventory influence the outcome?

## 10. Partial loss of flow

Learning objective: Determine how progressively reduced coolant flow affects protection-system action and peak temperature.

Activity: Manually reduce pump flow to 75%, 50%, 25% and 0%. Note whether and when the automatic trip occurs.

Record or compare: Minimum coolant flow and corresponding peak cladding temperature.

Plot: peak cladding temperature versus minimum coolant flow

## 11. Failure of automatic reactor trip during flow reduction

Learning objective: Understand the protective role of low-flow and high-temperature trip signals.

Activity: Disable automatic reactor trip, reduce coolant flow and manually SCRAM before exceeding the assigned classroom temperature limit.

Instructor note: This is a diagnosis-and-response activity, not a realistic safety calculation.

# D. Loss-of-heat-sink exercises

## 12. Loss of main heat sink

Learning objective: Follow the sequence from loss of secondary heat removal to primary-system heat-up and pressurization.

Activity: Use the LOHS preset. Observe reactor trip, coolant temperature, pressure, auxiliary feedwater response, spray and PORV action.

## 13. Auxiliary feedwater recovery

Learning objective: Evaluate how auxiliary feedwater capacity influences recovery from loss of the main heat sink.

Activity: Initiate LOHS and repeat the case with AFW at 0%, 25%, 50%, 75% and 100%.

Record or compare: Maximum primary pressure, maximum coolant temperature, maximum cladding temperature and time to stabilize.

Key lesson: Determine the minimum AFW setting that produces recovery in the teaching model.

## 14. Pressure-control comparison

Learning objective: Distinguish pressure control from decay-heat removal.

Activity: Following LOHS, compare spray only, PORV only, spray plus PORV, AFW without pressure control, and AFW with pressure control.

Key lesson: Pressure-control systems may reduce pressure, but they do not replace the need for decay-heat removal.

# E. Small-break LOCA exercises

## 15. Standard SBLOCA sequence

Learning objective: Identify the main stages of a small-break loss-of-coolant accident and its recovery.

Activity: Use the SBLOCA preset or the built-in SBLOCA recovery demonstration.

Record or compare: Identify the break opening, inventory reduction, depressurization, trip, persistence of decay heat, ECCS response, inventory recovery and transition to long-term cooling.

## 16. Effect of break size during SBLOCA

Learning objective: Relate break size to depressurization, coolant loss and peak cladding temperature.

Activity: Run cases at break settings of 5%, 10%, 15%, 20% and 30%.

Record or compare: Depressurization rate, inventory-loss rate, ECCS initiation time, minimum inventory and peak cladding temperature.

Plot: maximum cladding temperature versus break size

## 17. Automatic versus manual ECCS

Learning objective: Compare automatic protection with operator-initiated emergency injection.

Activity: Run the same SBLOCA twice: first with Auto ECCS enabled, then with Auto ECCS disabled and manual injection.

Record or compare: ECCS start time, minimum inventory, peak cladding temperature and stabilization time.

## 18. ECCS injection timing

Learning objective: Understand that ECCS effectiveness depends on system pressure and initiation timing.

Activity: For one SBLOCA, manually initiate ECCS immediately, below 12 MPa, at 80% inventory and after the high-cladding-temperature alarm.

Record or compare: Compare injection effectiveness, minimum inventory and peak cladding temperature.

## 19. Controlled depressurization

Learning objective: Evaluate the trade-off between reducing pressure to improve ECCS delivery and losing additional inventory through relief paths.

Activity: During SBLOCA recovery, compare no PORV action, moderate PORV opening and excessive PORV opening.

Record or compare: Pressure history, ECCS effectiveness, PORV inventory loss and peak cladding temperature.

## 20. Core uncovery and cladding-temperature response

Learning objective: Explain why heat transfer degrades as primary inventory falls and the core becomes less effectively covered.

Activity: Allow inventory to fall below approximately 75% and observe the cladding-temperature response.

Questions for discussion

Why does cladding temperature increase rapidly as inventory decreases?

Why does reduced core coverage weaken heat transfer?

Why is restoring inventory central to LOCA mitigation?

# F. Large-break LOCA exercises

## 21. Standard LBLOCA response

Learning objective: Trace rapid depressurization, coolant loss, emergency injection and reflood following a large break.

Activity: Use the LBLOCA preset or the LBLOCA ECCS response demonstration.

Record or compare: Pressure, inventory, pump flow, ECCS response, cladding temperature and long-term cooling status.

## 22. SBLOCA versus LBLOCA comparison

Learning objective: Compare the distinct timing and system challenges created by small and large breaks.

Activity: Run both presets under comparable initial conditions and complete the comparison table below.

| Parameter | SBLOCA | LBLOCA |
| --- | --- | --- |
| Depressurization rate | Lower | Higher |
| Inventory-loss rate | Lower | Higher |
| Time available for action | Longer | Shorter |
| High-pressure period | Longer | Shorter |
| Low-pressure ECCS availability | Later | Earlier |
| Reflood demand | Moderate | Strong |

Discussion: Explain why the largest break does not necessarily produce the highest pressure, but generally causes the fastest inventory challenge.

## 23. ECCS capacity during LBLOCA

Learning objective: Determine how emergency injection capacity affects reflood and coolant-inventory recovery.

Activity: Repeat an LBLOCA with ECCS set to 25%, 50%, 75% and 100%.

Record or compare: Classify each case as continuing inventory loss, stable low inventory, gradual reflood or full recovery.

## 24. Break isolation study

Learning objective: Evaluate the benefit of reducing or isolating the break during the recovery phase.

Activity: Run an LBLOCA and reduce the break size at 30 s, 60 s and 120 s; compare with a case in which it is not isolated.

Record or compare: Minimum inventory, peak cladding temperature and time to stable cooling.

# G. Station-blackout exercises

## 25. Standard station blackout

Learning objective: Recognize the combined consequences of reactor trip, loss of forced circulation and restricted active safety-system availability.

Activity: Use the SBO preset and observe trip, primary flow, heat removal, ECCS availability, pressure and temperature evolution.

## 26. SBO coping time

Learning objective: Introduce the concept of coping time before active cooling is restored.

Activity: Run an SBO and restore cooling after 30 s, 60 s, 120 s and 180 s.

Record or compare: Maximum cladding temperature, maximum pressure and minimum coolant inventory before recovery.

## 27. Recovery from SBO

Learning objective: Identify the staged progression from blackout conditions to stable shutdown cooling.

Activity: Use the built-in Station blackout recovery demonstration.

Record or compare: Identify reactor trip, loss of pumps and injection, limited coping cooling, restoration of power, ECCS recovery, controlled depressurization and RHR initiation.

## 28. Comparison of SBO and loss of heat sink

Learning objective: Explain why station blackout can be more challenging than loss of the main heat sink alone.

Activity: Compare SBO and LOHS cases under similar initial conditions.

Record or compare: Available flow, active injection, heat removal, pressure response, temperature response and recovery options.

# H. Residual heat-removal exercises

## 29. RHR pressure permissive

Learning objective: Demonstrate that residual heat-removal systems become useful only after suitable pressure reduction.

Activity: After reactor trip, attempt to use RHR at high pressure. Depressurize the primary system and observe when RHR begins to remove significant heat.

Record or compare: Pressure at which RHR response becomes visible and the subsequent coolant-temperature trend.

## 30. Transition to long-term cooling

Learning objective: Develop a justified sequence for moving from an accident transient to stable shutdown cooling.

Activity: Create and execute a sequence that includes reactor trip, inventory protection, secondary heat removal, controlled depressurization, RHR initiation and eventual reduction of ECCS.

Questions for discussion

Why is each action required?

Which actions must occur before RHR can contribute?

What indicators show that stable long-term cooling has been achieved?

# I. Instrumentation and data-analysis exercises

## 31. Instrument noise and signal interpretation

Learning objective: Distinguish real transient behaviour from measurement fluctuation.

Activity: Increase the instrument-noise setting and compare the instantaneous indicated power with the general power trend and thermal response.

## 32. Strip-chart interpretation

Learning objective: Develop the ability to reconstruct a transient from recorded trends.

Activity: Provide students with an unlabeled transient record.

Record or compare: Identify the initiating event, trip time, ECCS start, minimum inventory, peak cladding temperature, recovery point and transition to RHR.

## 33. CSV transient analysis

Learning objective: Use exported simulator data for quantitative post-run analysis.

Activity: Log a complete transient and analyse the CSV file in Excel, Python, MATLAB or Octave.

Record or compare: Time, scenario, power, decay heat, temperatures, pressure, inventory, reactivity, flow, trip status and operator-control histories.

Inventory-loss rate = ΔM / Δt

Depressurization rate = ΔP / Δt

Peak cladding temperature = max[Tclad(t)]

ECCS response time = tECCS on - tfault

## 34. Alarm and trip chronology

Learning objective: Build a defensible event chronology using the strip chart and CSV record.

Activity: For one transient, complete an event table containing the time and evidence for each major event.

| Event | Time | Trigger or observation |
| --- | --- | --- |
| Fault initiated |  |  |
| Reactor trip |  |  |
| Pressure below ECCS threshold |  |  |
| ECCS active |  |  |
| Minimum inventory |  |  |
| Peak cladding temperature |  |  |
| RHR effective |  |  |
| Stable cooling |  |  |

# J. Open-ended operator exercises

## 35. Stabilize the plant challenge

Learning objective: Integrate diagnosis, prioritization and control actions during an unknown transient.

Activity: The instructor introduces an unknown event by changing one or more controls. Students must diagnose the event, trip if necessary, protect inventory, maintain heat removal, control pressure and establish long-term cooling.

Record or compare: Correct diagnosis, response time, peak cladding temperature, minimum inventory, PORV use and successful entry into RHR.

## 36. Minimum-action recovery

Learning objective: Develop efficient recovery strategies without unnecessary control actions.

Activity: Recover from an SBLOCA or LOHS using the smallest practical number of operator actions, then justify every action taken.

## 37. Competing-objectives exercise

Learning objective: Recognize that operator actions may create both benefits and penalties.

Activity: Recover from an SBLOCA while maintaining cladding temperature below an assigned value, inventory above an assigned value, pressure below the RHR-entry condition, limited PORV use and a stable final temperature trend.

## 38. Blind diagnosis exercise

Learning objective: Use trend evidence to identify a transient without being told the scenario.

Activity: Hide the scenario name or let one student introduce a fault while another observes only the displays.

Record or compare: Possible diagnoses include LOFA, LOHS, SBLOCA, LBLOCA, SBO, excessive PORV opening, insufficient ECCS and loss of AFW.

Key lesson: Students should state the evidence that supports their diagnosis, not merely name the event.

# K. Recommended core laboratory set

A coherent introductory sequence for a normal undergraduate thermal-hydraulics course.

| No. | Core exercise | Primary learning outcome |
| --- | --- | --- |
| 1 | Normal steady-state heat balance | Relate power generation, heat transfer and heat removal. |
| 2 | Effect of coolant flow | Explain how reduced flow increases thermal resistance and component temperature. |
| 3 | SCRAM and decay-heat removal | Distinguish neutron shutdown from continuing residual heat generation. |
| 4 | Loss-of-flow accident | Trace protection action, coastdown and natural-circulation cooling. |
| 5 | Loss of heat sink and AFW recovery | Connect secondary heat removal to primary temperature and pressure control. |
| 6 | SBLOCA: automatic versus manual ECCS | Evaluate timing, pressure dependence and inventory recovery. |
| 7 | SBLOCA versus LBLOCA | Compare accident timescales, depressurization and reflood demand. |
| 8 | Station blackout and long-term cooling | Integrate coping, recovery, depressurization and RHR entry. |

Suggested assessment dimensions

Correct identification of the initiating event and relevant system trends.

Appropriate prioritization of reactor trip, inventory control and heat removal.

Use of quantitative evidence from the strip chart or CSV record.

Explanation of cause-and-effect relationships and unintended consequences of actions.

Clear statement of the simulator limitations and avoidance of plant-specific safety claims.

Closing note

The exercises should be introduced progressively: first normal heat-transfer behaviour, then reactor trip and residual heat, followed by single-failure transients, loss-of-coolant accidents and finally open-ended diagnosis. This progression allows the simulator to function as both a guided visualization tool and a quantitative laboratory platform.