| OPEN NUCLEAR ENGINEERING TEACHING SUITE Core Loading and Fuel Management Simulator What it is, what it teaches, and how the core physics is deliberately simplified TEACHING PRESENTATION BRIEF | Nuclear Engineering Programme Universiti Teknologi Malaysia |
| --- | --- |

| Source basis simulators/CoreLoadingSimulator/core_loading_thorium_poc_fixed.py - current implementation | Scope Current educational implementation |
| --- | --- |

|  | CORE LOADING AND FUEL MANAGEMENT SIMULATOR | 02 |
| --- | --- | --- |

1. What it is

| Core identity An interactive 11 x 11 assembly-level core-loading and fuel-management teaching simulator. A rounded PWR-like lattice with conventional and thorium-bearing assembly choices. A coupled two-group diffusion, power, depletion, feedback, poison, control, and refueling demonstration. | Intended learning role Makes loading-pattern effects on flux, power, leakage, peaking, tilt, boron, and shutdown margin visible. Connects assembly position and identity to burnup, isotope inventories, outage decay, and multi-cycle history. Supports pattern studies, fuel-cycle comparisons, guided refueling, and CSV/JSON-based analysis. |
| --- | --- |

Teaching position: The simulator bridges hand loading-pattern exercises and professional lattice/core-design and fuel-management systems.

|  | CORE LOADING AND FUEL MANAGEMENT SIMULATOR | 03 |
| --- | --- | --- |

2. What it does

| Build loading patterns Place fresh, burned, absorber-bearing, thorium, seed, reflector, or empty assemblies on the editable map. | Solve the core Calculate fast/thermal flux, fission power, k-effective, leakage, boron demand, peaking, tilt, and centroid offset. |
| --- | --- |
| Advance a cycle Step full-power days to evolve burnup, U-235, Pu-239, absorber, Pa-233/U-233, Pm-149/Sm-149, and histories. | Study feedback & control Inspect fuel/moderator temperature, density, equilibrium Xe-135, bank worth, all-rods-in k, and shutdown margin. |
| Manage assemblies Move, swap, discharge, store, reload, and preserve permanent IDs, isotopes, burnup, completed cycles, and history. | Plan refueling Use staging racks, validation, undo/redo, EOC-to-BOC preview, outage decay, and transactional commit. |

|  | CORE LOADING AND FUEL MANAGEMENT SIMULATOR | 04 |
| --- | --- | --- |

3. What it doesn't do

| Outside the model No pin-cell or pin-power calculation, detailed assembly heterogeneity, thermal-hydraulic channel model, or 3-D axial solution. No continuous-energy transport, evaluated-library cross-section processing, transport correction, or uncertainty analysis. No detailed depletion chain, soluble-boron chemistry, control-rod geometry, fuel performance, or safety-limit calculation. No seconds-scale kinetics, startup/shutdown xenon transient, spatial xenon oscillation, or accident progression. | Outside its authority Cross sections, feedbacks, depletion rates, control absorptions, and limits are classroom-calibrated. The 11 x 11 map and symmetric 29-position bank are generic, not a plant loading pattern. Not suitable for reactor design, licensing, criticality safety, operator qualification, or real fuel management. Results support relative comparisons inside this model, not predictions of a physical core. |
| --- | --- |

Boundary of use: The model teaches spatial coupling and fuel-management logic; it does not establish nuclear-design or safety margins.

|  | CORE LOADING AND FUEL MANAGEMENT SIMULATOR | 05 |
| --- | --- | --- |

4. Physics included

| Two-group core solve Coupled fast and thermal finite-difference diffusion k-eigenvalue equations on assembly-homogenized cells. Reflectors participate in leakage/reflection but generate no fission power or burnup. | Power & criticality control Assembly power follows fission rate without empirical profile compression. An iterative soluble-boron search approaches operating k = 1 when excess reactivity is available. |
| --- | --- |
| Depletion & breeding Full-power-day steps evolve U-235, Pu-239, burnable absorber, Pa-233, U-233, Pm-149, and Sm-149. | Feedback & poisons Local power iterates with fuel temperature, moderator temperature/density, and equilibrium full-power Xe-135. Pm/Sm inventories evolve across cycle steps and outages. |
| Control banks One insertion command applies absorber to a symmetric 29-position pattern. Indicators compare unrodded k, inserted worth, all-banks-in k, and illustrative shutdown margin. | Assembly lifecycle Permanent IDs carry isotopes, burnup, history, and cycle credit through moves, storage, outage decay, and reload. |

|  | CORE LOADING AND FUEL MANAGEMENT SIMULATOR | 06 |
| --- | --- | --- |

5. Physics simplified

| Reduced-order representation Each core cell is one homogenized assembly; axial shape, pins, grids, baffles, and detailed reflectors are absent. Cross sections and depletion relationships are compact classroom datasets, not generated from evaluated nuclear data. Feedback temperatures and density are algebraic steady-state fields rather than coupled channel thermal hydraulics. The soluble-boron search changes thermal absorption through an illustrative equivalent worth. Control banks share one insertion percentage and predefined symmetric positions. | Cycle-scaled physics Each step is a spatially coupled steady state followed by an exposure update, not a transient kinetics integration. Xe-135 is an equilibrium full-power field, so hourly startup, shutdown peaks, and oscillations are not represented. Outage decay analytically transfers Pa-233 to U-233 and Pm-149 to Sm-149 while preserving long-lived inventories. Display interpolation and edge falloff affect only graphics and never feed back into the solved assembly values. |
| --- | --- |

|  | CORE LOADING AND FUEL MANAGEMENT SIMULATOR | 07 |
| --- | --- | --- |

6. Pedagogical decisions taken

| Assembly-scale visibility An 11 x 11 map keeps leakage, symmetry, tilt, peaking, and batch placement visible during a class exercise. | Direct eigenvalue meaning Reported k is the eigenvalue of the discretized two-group equations, not a transformed reactivity index. |
| --- | --- |
| Critical operation view Uncontrolled k is paired with a boron-controlled operating k and a boron requirement that trends toward EOC. | Persistent fuel identity Permanent IDs separate physical assemblies from positions so state and history follow moves and reloads. |
| Transactional refueling Draft staging, validation, undo/redo, EOC-to-BOC preview, and commit make refueling decisions reviewable. | Multiple evidence views Maps, 3-D surfaces, trends, power-change views, inventories, JSON state, and CSV export support comparison. |

Current-code clarification: The smooth 3-D surface is a visualization of solved assembly-centre values. Interpolation, outside-core falloff, and camera settings do not alter the diffusion, depletion, or criticality calculations.