THREE SIMULATORS, ONE TEACHING SUITE

Open Nuclear Engineering
Teaching Simulators

Scope, physics, simplifications and pedagogical choices

Reactor Physics  •  Thermal-Hydraulics & LOCA  •  Core Loading & Fuel Management

ORIENTATION

1. What they are

| Reactor Physics & Kinetics A zero-dimensional kinetics and controls laboratory, from classroom-scaled response to a dimensioned Representative LWR profile. | Thermal-Hydraulics & LOCA A conserved lumped-system model of power, decay heat, coolant mass/energy, pressure, boiling, CHF and safety-system response. | Core Loading & Fuel Management An assembly-level 11 x 11 two-group diffusion, depletion, feedback, control-bank and multi-cycle refueling laboratory. |
| --- | --- | --- |

Shared identity: Interactive Python desktop teaching tools with plots, scenarios and exportable evidence. They span different time and spatial scales.

LEARNING INTERACTIONS

2. What they do

| Seconds-scale kinetics Explore reactivity balance, delayed neutrons, poisons, power/heat, feedback, startup, SCRAM and faults. | Accident-response trends Explore inventory, stored energy, saturation, boiling/CHF, breaks, heat sinks, ECCS, AFW, spray and RHR. | Cycle and spatial studies Explore loading patterns, eigenvalue, power shape, leakage, burnup, breeding, poisons, control and refueling. |
| --- | --- | --- |

Teaching workflow: Predict, select a controlled case, change one factor, observe coupled response, export evidence, explain limitations.

SCOPE BOUNDARY

3. What they do not do

No engineering prediction: Results are qualitative and classroom-calibrated; plausible values are not evidence of plant accuracy.

No design or licensing: Not for safety analysis, core design, criticality safety, operating limits, procedures or regulatory submissions.

No full fidelity: No pin/axial detail, full plant geometry, multidimensional two-phase flow, evaluated-data processing or validated component models.

No operator qualification: Interfaces and scenarios teach relationships; they are not full-scope simulators or real-time decision tools.

MODEL CONTENT

4. Physics included

| Reactor Six-group point kinetics; four prompt presets; I/Xe and Pm/Sm; decay heat; detailed reactivity balance; source-range startup; finite SCRAM; optional 3000 MWth energy balance. | Thermal-hydraulics Point kinetics and decay heat; fuel/clad nodes; coolant mass and stored energy; enthalpy flows; saturation pressure; boiling regimes/CHF; breaks and engineered heat removal. | Core loading Two-group diffusion k-eigenvalue; fission-rate power; depletion/breeding; thermal feedback; equilibrium Xe and evolving Sm; boron search; symmetric control banks; assembly tracking. |
| --- | --- | --- |

DELIBERATE ABSTRACTION

5. Physics simplified

Reactor: One point core; equivalent coefficients and banks; compressed poison/exposure options; dimensioned parameters remain declared teaching references.

Thermal-hydraulics: One primary volume; compact saturation table; coefficient-based critical flow, CHF, void, core coverage and safety-system effectiveness.

Core loading: One homogenized cell per assembly; 2-D only; compact cross sections/depletion; algebraic feedback fields; equilibrium full-power xenon.

Across the suite: Bounds, smooth transitions and controlled numerical schemes prioritize stable, interpretable comparisons.

WHY THE MODELS ARE OBSERVABLE

6. Pedagogical decisions taken

| Selectable kinetics timescales Reactor Lambda ranges from 0.080 s to 2e-5 s. Realistic-scale response triggers internal RK4 substeps rather than changing animation speed. | Compressed slow phenomena Poison and exposure demonstrations can fit a class; active settings and provenance are exported so results remain interpretable. | Evidence-rich safety physics Thermal-hydraulics exposes saturation, regime, CHF and event transitions while retaining transparent mass/energy accounting. | Visible spatial trade-offs Core maps, power-change views, trends, permanent IDs and transactional refueling make fuel-management decisions reviewable. |
| --- | --- | --- | --- |

TEACHING SEQUENCE

7. Choosing the right simulator

| Start with reactor physics Use when the question begins with reactivity, neutron population, period, poisoning, feedback, power control or SCRAM. | Move to thermal-hydraulics Use when the question is heat removal, pressure, inventory, boiling, CHF, LOCA, ECCS or accident recovery. | Use core loading for cycles Use when the question is spatial power, leakage, loading pattern, depletion, breeding, control worth, boron or refueling. |
| --- | --- | --- |

Best practice: Compare trends with course theory, record non-default settings, and state which omitted physics a higher-fidelity code would require.