STUDENT EXPERIMENT GUIDE

Reactor Physics & Kinetics

Seven guided investigations of reactivity, kinetics, feedback and control

Observe • Predict • Test • Explain

Open Nuclear Engineering Teaching Suite

# How to use this guide

One change at a time: Reset between trials and vary one main input unless the experiment says otherwise.

Make a prediction: Write the expected direction and timing of each response before pressing Start.

Use evidence: Read trends from plots and export CSV data when quantitative comparison is requested.

Repeatability: Instrument noise means two runs may differ slightly. Repeat when necessary.

Scope: Treat values as classroom-scale model outputs, not plant predictions or safety limits.

## Main controls used

Rod position and ±5 pcm manual reactivity steps

Instructor kinetics/poison/physics presets; soluble boron, coolant flow and heat sink

Manual, automatic power-hold and load-follow modes

SCRAM, selectable faults, fault severity and instrument noise

## Suggested assessment evidence

Prediction table or annotated trend sketch

CSV plot with labelled event times

Short physical explanation using reactivity balance

A limitations statement distinguishing model behavior from plant behavior

EXPERIMENT 1

# Small reactivity steps and reactor period

| Objective | Relate reactivity insertion to power response and reactor period. |
| --- | --- |
| Before running | Predict how +5, +10 and −5 pcm changes will affect initial power slope and period. |
| Suggested setup | Reset; Manual mode; 100% flow and heat sink; default rod/boron; noise near 0–1%. |

## Procedure

Start and allow the initial condition to settle.

Apply one +5 pcm step. Record power and period for 20–30 s.

Apply a second +5 pcm step; observe the changed slope.

Reset and repeat with one −5 pcm step.

Export the run or copy values at consistent times.

## Record and compare

Reactivity (pcm), power (%) and period (s) immediately before and after each step.

Time required for power to change by a selected amount, such as 5 percentage points.

## Analysis questions

1. Why does positive reactivity produce a positive power slope?

2. Is the response proportional to step size in this bounded model?

3. How do delayed-neutron precursors make the response different from an instantaneous jump?

## Extension

Challenge: Repeat with instrument noise increased and discuss whether the inferred period is still reliable.

EXPERIMENT 2

# Control rods versus soluble boron

| Objective | Compare two mechanisms that change total reactivity. |
| --- | --- |
| Before running | Predict which direction rod withdrawal and boron addition move reactivity and power. |
| Suggested setup | Manual mode; normal flow/heat sink; low noise; reset between trials. |

## Procedure

Run a baseline trial and note total, rod, boron and xenon reactivity terms.

Withdraw the rod bank by a small fixed amount; record the transient.

Reset. Increase soluble boron by a fixed amount; record the transient.

Adjust rods to compensate for the added boron and approach criticality.

Compare control changes that produce similar total reactivity.

## Record and compare

Rod position, boron ppm, each reactivity contribution, total reactivity and power.

Approximate rod change required to offset the selected boron change.

## Analysis questions

1. Why can two different control settings produce similar total reactivity?

2. Which displayed term makes the reactivity balance easiest to explain?

3. Why should simulator worths not be treated as plant calibration data?

EXPERIMENT 3

# Negative temperature feedback

| Objective | Observe how thermal conditions feed back into reactivity. |
| --- | --- |
| Before running | Predict the sequence after reducing coolant flow or heat-sink effectiveness. |
| Suggested setup | Manual mode; stable power; default boron; low noise. |

## Procedure

Record initial fuel, cladding and coolant temperatures plus temperature reactivity.

Reduce coolant flow moderately while keeping heat sink fixed.

Observe temperatures, temperature reactivity and neutron power until a clear trend appears.

Reset and repeat by reducing heat sink instead of flow.

Compare the order and magnitude of responses.

## Record and compare

Fuel/clad/coolant temperatures, temperature-reactivity term and power versus time.

Event time for control change and any automatic trip.

## Analysis questions

1. Why does heating insert negative reactivity in the model?

2. Why can heat power and neutron power separate during a transient?

3. How do the three thermal time constants shape the lag?

## Extension

Challenge: Try a milder and a more severe flow reduction; identify when protection action dominates the natural feedback.

EXPERIMENT 4

# SCRAM and decay heat

| Objective | Distinguish fission power from total heat and residual decay heat. |
| --- | --- |
| Before running | Predict which variable falls fastest after SCRAM and which remains significant. |
| Suggested setup | Reach a stable operating condition near full power; normal cooling. |

## Procedure

Start and run long enough to establish a baseline.

Press SCRAM and mark the event time.

Observe true neutron power, total heat and decay heat for at least 60 s.

Keep cooling normal; then repeat with a reduced heat sink after SCRAM.

Export both trials for overlay plotting.

## Record and compare

Normalized neutron power, total heat, decay heat and fuel temperature.

Initial and later decay-heat percentages after SCRAM.

## Analysis questions

1. Why does neutron power collapse faster than heat generation?

2. Why must cooling continue after shutdown?

3. How do the classroom-scaled decay groups affect the duration of the exercise?

EXPERIMENT 5

# Iodine–xenon poisoning and load following

| Objective | Explore delayed poisoning during changing power demand. |
| --- | --- |
| Before running | Predict when xenon will rise or fall during a load-follow cycle. |
| Suggested setup | Reset; select Load-follow demo; normal cooling; low noise. |

## Procedure

Start load-follow mode and observe target and actual power.

Record iodine index, xenon index and xenon reactivity at regular intervals.

Identify a rising-power and falling-power portion of the cycle.

Pause at two contrasting points and compare the reactivity balance.

Explain the rod motion needed to maintain power as xenon changes.

## Record and compare

Target/actual power, rod position, iodine, xenon and xenon reactivity.

Timing of xenon extrema relative to power extrema.

## Analysis questions

1. Why does xenon response lag power?

2. How does xenon burnout depend on power in the model?

3. What real-time scaling has been deliberately compressed for teaching?

## Extension

Challenge: Repeat in automatic power-hold mode after a power change and compare controller action.

EXPERIMENT 6

# Fault diagnosis from conflicting indications

| Objective | Use physical redundancy and trends to diagnose instrumentation or actuator faults. |
| --- | --- |
| Before running | Predict signatures of detector bias, frozen detector, stuck rod and pump trip. |
| Suggested setup | Choose one fault at a time; begin at low severity; do not reveal the chosen fault to the diagnosing student. |

## Procedure

Student A selects and activates one fault; Student B observes without seeing the selection.

Apply a small, known control change and watch measured power, true power, temperatures and reactivity.

State a diagnosis and cite at least three pieces of evidence.

Reveal the fault; reset and repeat with a different fault.

Increase severity and discuss whether diagnosis becomes easier or introduces trips.

## Record and compare

Measured versus true power, rod command/position, flow, temperatures, alarms and event log.

Correct diagnosis, evidence used and rejected alternative explanations.

## Analysis questions

1. Which faults create disagreement between measured and true power?

2. How can thermal response validate or challenge a neutron-power indication?

3. Why is this exercise conceptual rather than operator qualification?

EXPERIMENT 7

# Representative LWR kinetics and energy conservation

| Objective | Compare the dimensioned LWR profile with the classroom-scaled model and test its declared thermal energy balance. |
| --- | --- |
| Before running | Predict how a much smaller prompt-generation time changes numerical treatment, and which MW heat flows balance at full power. |
| Suggested setup | Pause; open Instructor: Pedagogical Settings; choose Representative LWR, full-power equilibrium, and restore normal flow/heat sink. |

## Procedure

Apply the profile and record rated power, prompt-generation time, integrator and kinetics substeps.

Run at equilibrium and export CSV data for true power, total heat, fuel-to-clad heat, clad-to-coolant heat, heat removed and energy residual.

Reduce the heat sink moderately while holding flow fixed; observe stored-energy and temperature changes.

Reset to Representative LWR equilibrium, initiate SCRAM and observe finite rod insertion plus neutron/decay-heat separation.

Restore validated classroom defaults and repeat one small reactivity step for qualitative comparison.

## Record and compare

Kinetics preset, Lambda, integration method/substeps, percent power and MWth.

Fuel/clad/coolant temperatures, three heat-transfer rates and thermal energy residual.

## Analysis questions

1. Why is RK4 substepping required for Lambda = 2e-5 s while the legacy presets retain Euler stepping?

2. At equilibrium, how do 2910 MW, 2970 MW and 3000 MW transfers reflect the 97/2/1% deposition split?

3. Why do dimensional units and a small residual demonstrate consistency but not plant validation?

## Extension

Compare Classroom, Intermediate, Advanced and Representative LWR presets using the same +20 pcm insertion, recording active settings in every CSV export.