STUDENT EXPERIMENT GUIDE

Thermal-Hydraulics & LOCA

Seven guided investigations of heat removal, inventory, pressure and safety response

Observe • Predict • Test • Explain

Open Nuclear Engineering Teaching Suite

# How to use this guide

One change at a time: Reset between trials and vary one main input unless the experiment says otherwise.

Make a prediction: Write the expected direction and timing of each response before pressing Start.

Use evidence: Read trends from plots and export CSV data when quantitative comparison is requested.

Repeatability: Instrument noise means two runs may differ slightly. Repeat when necessary.

Scope: Treat values as classroom-scale model outputs, not plant predictions or safety limits.

## Main controls used

Primary flow, steam-generator heat removal, LOCA break size, boiling regime and CHF ratio

ECCS, auxiliary feedwater, PORV, spray, heaters and RHR

Automatic trip and automatic ECCS options

Normal, SBLOCA, LBLOCA, LOFA, LOHS and SBO presets; scripted recovery demos

## Suggested assessment evidence

Prediction table with response ordering

CSV plot of pressure, inventory and temperatures

Energy/inventory balance explanation

Comparison of protection response with and without a selected system

EXPERIMENT 1

# Primary flow and natural-circulation surrogate

| Objective | Relate pump flow to heat transfer and core temperatures. |
| --- | --- |
| Before running | Predict fuel, cladding and coolant responses as pump setting decreases. |
| Suggested setup | Normal scenario; 100% SG heat removal; no break; automatic trip initially off for comparison. |

## Procedure

Run at 100% pump until stable.

Reduce pump to 70%, then 40%, allowing a trend at each setting.

Record effective flow and three temperatures.

Reset; set pump to 0% and observe the residual natural-circulation surrogate.

Repeat with automatic trip on and compare.

## Record and compare

Pump setting, effective flow, power, fuel/clad/coolant temperatures and trip status.

Response lag and peak temperature for each trial.

## Analysis questions

1. Why does effective flow not fall exactly to zero when pumps stop?

2. Which thermal node responds fastest?

3. How does the lumped model simplify real natural circulation and flow distribution?

EXPERIMENT 2

# Loss of heat sink and auxiliary feedwater

| Objective | Test how secondary-side heat removal controls primary temperature and pressure. |
| --- | --- |
| Before running | Predict the effect of reducing SG heat removal and then adding AFW. |
| Suggested setup | Normal operation; full pump flow; no break; normal inventory. |

## Procedure

Record a steady baseline.

Reduce SG heat removal sharply and observe coolant temperature and pressure.

Add AFW in steps while keeping SG setting fixed.

Find an AFW setting that arrests or reverses the trend.

Reset and run the Loss of Heat Sink preset for comparison.

## Record and compare

SG and AFW settings, heat removal trend, coolant temperature, pressure and clad temperature.

Time of each intervention and resulting slope change.

## Analysis questions

1. Why does loss of secondary heat removal affect primary pressure?

2. How is AFW represented in the model?

3. Why is a recovered trend not proof of real equipment adequacy?

EXPERIMENT 3

# Small-break versus large-break LOCA

| Objective | Compare how break size changes depressurization, inventory loss and temperature response. |
| --- | --- |
| Before running | Rank the expected rates of pressure decrease, inventory loss and clad heating. |
| Suggested setup | Use SBLOCA and LBLOCA presets; keep automatic trip and ECCS enabled; use the same simulation speed. |

## Procedure

Run SBLOCA and export the full transient.

Reset; run LBLOCA for the same simulated duration and export.

Plot pressure and inventory on matched time axes.

Compare ECCS activation, power, decay heat and clad-temperature response.

Identify where model clamps or scripted settings influence the comparison.

## Record and compare

Pressure, inventory, break size, ECCS, clad temperature and trip status.

Initial slopes and selected values at equal times.

## Analysis questions

1. Why does a larger break generally depressurize and deplete inventory faster?

2. Why can lower pressure improve the model’s ECCS effectiveness?

3. Which real two-phase phenomena are absent from this comparison?

EXPERIMENT 4

# ECCS pressure dependence

| Objective | Investigate the model’s continuous classroom-scale ECCS effectiveness curve. |
| --- | --- |
| Before running | Predict injection effectiveness above 14 MPa, between 14–8 MPa, between 8–2 MPa and below 2 MPa. |
| Suggested setup | Manual ECCS control; automatic ECCS off; choose a controlled depressurization method such as a modest break or PORV. |

## Procedure

Establish a baseline with ECCS at a fixed setting.

Depressurize and note inventory slope near 15, 11, 5 and below 2 MPa.

Keep the ECCS slider unchanged across observations.

Compare injection effectiveness inferred from inventory behavior.

Repeat at a second ECCS command to check the qualitative relationship.

## Record and compare

Pressure, ECCS command, inventory and inventory slope at the four pressure regions.

Break/PORV setting and coolant temperature as confounding variables.

## Analysis questions

1. Why does the implementation use interpolation rather than step changes?

2. Why are 14, 8 and 2 MPa teaching reference points rather than plant setpoints?

3. What additional data would a real pump/injection model require?

EXPERIMENT 5

# Shutdown cooling sequence: decay heat, depressurization and RHR

| Objective | Demonstrate why shutdown heat removal depends on both residual heat and system conditions. |
| --- | --- |
| Before running | Predict when RHR becomes effective in the model and what happens if cooling is delayed. |
| Suggested setup | Stable normal operation; trip reactor; maintain adequate inventory. |

## Procedure

Initiate a trip and observe fission power versus decay heat.

Use SG/AFW to remove heat while pressure remains high.

Depressurize carefully using the model controls.

Apply RHR at high pressure and then below its availability range.

Compare coolant/clad-temperature slopes before and after RHR becomes effective.

## Record and compare

Power, decay heat, pressure, inventory, RHR setting and temperatures.

Times of trip, depressurization and effective RHR cooling.

## Analysis questions

1. Why does decay heat remain after trip?

2. Why is RHR deliberately restricted mainly to low pressure?

3. What parts of a real cooldown procedure are not represented?

EXPERIMENT 6

# Station blackout coping and recovery

| Objective | Evaluate the qualitative progression from loss of powered systems to restored heat removal. |
| --- | --- |
| Before running | Predict the first variables to deteriorate and which recovery action is most influential. |
| Suggested setup | Use the Station Blackout preset or scripted SBO recovery demo. |

## Procedure

Run the SBO case and mark the loss-of-power event.

Observe pump flow, SG heat removal, decay heat, pressure, inventory and temperatures.

Run once without intervention for a fixed duration.

Reset and follow a recovery strategy using available AFW/surrogate heat removal and later RHR as conditions permit.

Compare peak temperature and minimum inventory across both trials.

## Record and compare

Timeline of system losses and operator/recovery actions.

Peak clad temperature, maximum pressure, minimum inventory and time to stable cooling.

## Analysis questions

1. Which stored-energy or decay-heat effects drive the transient?

2. Which intervention changes the trend first in this model?

3. Why must the result not be interpreted as an SBO coping-time calculation?

## Extension

Challenge: Compare the scripted demo with a student-designed recovery and critique which steps are model-driven rather than plant-realistic.

EXPERIMENT 7

# Boiling regimes, CHF and conserved coolant energy

| Objective | Relate coolant energy, saturation, void, core coverage and CHF margin during a controlled loss-of-cooling transient. |
| --- | --- |
| Before running | Predict the order of single-phase, nucleate-boiling, transition-boiling and film-boiling indications as heat removal degrades. |
| Suggested setup | Normal scenario; automatic trip initially enabled; use a controlled reduction in primary flow or SG heat removal and log the event timeline. |

## Procedure

Record the initial mass, stored energy, pressure, saturation temperature, boiling regime, void and CHF ratio.

Reduce one cooling parameter in steps while keeping other controls fixed; mark every regime or CHF event.

Observe how inventory/core coverage modifies cladding-to-coolant heat transfer.

Reset and repeat with an earlier recovery action such as restored flow, SG/AFW heat removal or ECCS when applicable.

Export both runs and align them by intervention time.

## Record and compare

Coolant mass/inventory, stored energy, bulk temperature, pressure and saturation temperature.

Boiling regime, void fraction, CHF ratio, clad temperature, boundary mass/enthalpy flows and timeline events.

## Analysis questions

1. Why can nucleate boiling enhance heat transfer before post-CHF degradation?

2. How do cold ECCS injection and break/PORV discharge affect both mass and stored energy?

3. Which missing spatial and two-fluid phenomena prevent use as a DNB/CHF or reflood prediction?

## Extension

Compare the same transient with automatic trip/ECCS enabled and disabled, using event timing rather than peak values alone.