| OPEN NUCLEAR ENGINEERING TEACHING SUITE Reactor Physics and Kinetics Simulator What it is, what it teaches, and how the physics is deliberately simplified TEACHING PRESENTATION BRIEF | Nuclear Engineering Programme |
| --- | --- |

| Source basis simulators/ReactorPhysicsSimulator/reactor_teaching_simulator.py - current verified implementation | Scope Current educational implementation |
| --- | --- |

|  | REACTOR PHYSICS AND KINETICS SIMULATOR | 02 |
| --- | --- | --- |

1. What it is

| Core identity An interactive Python/Tkinter teaching simulator for reactor operation, startup, and transient response. A generic thermal-reactor / PWR-like point model rather than a replica of a named plant. A coupled kinetics, poisons, decay-heat, thermal-feedback, instrumentation, protection, and exposure demonstration. | Intended learning role Makes reactivity components, neutron level, period, precursors, poisons, temperatures, and exposure visible. Lets instructors compare classroom and advanced physics profiles without changing applications. Supports demonstrations, guided laboratories, startup studies, fault diagnosis, and CSV analysis. |
| --- | --- |

Teaching position: The simulator bridges textbook point kinetics and higher-fidelity spatial or plant-analysis software.

|  | REACTOR PHYSICS AND KINETICS SIMULATOR | 03 |
| --- | --- | --- |

2. What it does

| Operate the reactor Move rods, apply manual reactivity, adjust boron, flow, heat sink, noise, source, and power setpoint. | Choose teaching fidelity Select prompt kinetics, poison timescale, load-follow period, initial condition, exposure state, and physics profile. |
| --- | --- |
| Observe coupled physics Track true/measured power, heat, period, temperatures, I/Xe, Pm/Sm, exposure, and separated reactivity terms. | Study startup & control Use source-, intermediate-, and power-range indications plus subcritical multiplication in the advanced profile. |
| Diagnose faults Inject rod, SCRAM, pump, heat-sink, detector, boron, controller, and protection-system faults. | Analyse results Use diagnostics, event history, strip charts, and CSV rows that retain active settings and model provenance. |

|  | REACTOR PHYSICS AND KINETICS SIMULATOR | 04 |
| --- | --- | --- |

3. What it doesn't do

| Outside the model No spatial flux, power-shape, pin-power, core-loading, or xenon-oscillation solution. No multigroup transport/diffusion or inventory-based depletion; exposure is a compressed teaching law. No primary-system pressure, inventory, boiling, two-phase flow, DNBR, CHF, or LOCA model. No mechanistic fuel performance, chemistry, cladding oxidation, or materials degradation. | Outside its authority Not calibrated to a specific reactor, detector system, rod bank, fuel cycle, or operating procedure. Not a design, licensing, safety-analysis, operator-qualification, or plant-control code. Kinetic constants, poison yields/worths, feedback coefficients, thresholds, and cycle scaling are illustrative. Verification supports numerical consistency and qualitative teaching trends, not plant prediction. |
| --- | --- |

Boundary of use: The model teaches relationships and controlled comparisons; it does not establish safety margins or real transient magnitudes.

|  | REACTOR PHYSICS AND KINETICS SIMULATOR | 05 |
| --- | --- | --- |

4. Physics included

| Neutron kinetics Six delayed-neutron-group point kinetics with four instructor presets, including a Representative LWR option. External-source startup and an independently verified reactor-period response. | Reactivity accounting Rod, manual, Doppler, moderator-temperature, density, void, boron, Xe, Sm, and cycle terms. A smooth integral rod-worth curve and live differential worth in the advanced profile. |
| --- | --- |
| Poisons & exposure Coupled I-135/Xe-135 and optional Pm-149/Sm-149 production, decay, burnout, and poisoning. | Decay heat Three decay-heat groups initialized consistently with the selected operating state. Total heat remains distinct from neutron power after SCRAM. |
| Thermal feedback Lumped fuel, cladding, and coolant temperatures with separated Doppler, moderator, density, and void terms; Representative LWR adds explicit MW/MJ conservation. Flow and heat-sink effectiveness drive the thermal response. | Instrumentation & protection Source/intermediate/power ranges, log neutron level, noise, detector faults, trips, SCRAM logic, and overrides. |

|  | REACTOR PHYSICS AND KINETICS SIMULATOR | 06 |
| --- | --- | --- |

5. Physics simplified

| Reduced-order representation The whole core remains one point, with no geometry, leakage distribution, or local feedback. Advanced rod worth is smooth and position-dependent, but still represents one equivalent bank. Boron and feedback coefficients are equivalent classroom parameters, not plant calibration data. Fuel, cladding, and coolant each use one representative temperature. Cycle exposure uses an illustrative linear excess-reactivity law rather than nuclide depletion. | Classroom-scaled dynamics Prompt kinetics can be Classroom (0.080 s), Intermediate (0.020 s), Advanced (0.005 s), or Representative LWR (2.0e-5 s). Poison half-times can be compressed for a lab or set to a reference trend; decay-heat times remain fixed. One simulated full-power second equals one EFPD only in the selected exposure demonstration. Explicit stepping, bounded settings, clipping, and state limits prioritize stable comparative learning. |
| --- | --- |

|  | REACTOR PHYSICS AND KINETICS SIMULATOR | 07 |
| --- | --- | --- |

6. Pedagogical decisions taken

| Selectable prompt response Four bounded Lambda presets expose timescale effects; Representative LWR adds 3000 MWth scaling and automatic RK4 substeps. | Verified integration Legacy presets use dt = 0.02 s explicit Euler; Representative LWR uses RK4 with automatic prompt-timescale substeps. Both have documented equilibrium and inhour checks. |
| --- | --- |
| Selectable slow physics I/Xe timescales and load-follow period are instructor choices; applying a choice resets consistent inventories. | Visible reactivity balance Advanced diagnostics separate Doppler, moderator, density, void, boron, Xe, Sm, rod, and cycle contributions. |
| Safe teaching bounds Preset-only constants, startup inhibit, automatic trips, reactivity clamps, and state bounds limit numerical excursions. | Traceable evidence Every CSV row stores physics profile, kinetics/poison settings, initial condition, exposure preset, states, and faults. |

Current-code clarification: Lambda is no longer a single fixed value. The validated default is 0.080 s. The 0.020 s and 0.005 s presets retain the legacy 0.02 s outer step; Representative LWR uses Lambda = 2.0e-5 s, about 33 RK4 kinetics substeps per outer step, finite SCRAM insertion, and a declared 3000 MWth energy balance.