| OPEN NUCLEAR ENGINEERING TEACHING SUITE Thermal-Hydraulics and LOCA Simulator What it is, what it teaches, and how the plant physics is deliberately reduced TEACHING PRESENTATION BRIEF | Nuclear Engineering Programme |
| --- | --- |

| Source basis simulators/ThermalHydraulicsSimulator/thermal_hydraulics_simulator.py - current verified implementation | Scope Current educational implementation |
| --- | --- |

|  | THERMAL-HYDRAULICS AND LOCA SIMULATOR | 02 |
| --- | --- | --- |

1. What it is

| Core identity An interactive Python/Tkinter model coupling reactor power, decay heat, fuel/cladding temperatures, coolant mass and energy, pressure, and inventory. A generic 3000 MWth, 15.5 MPa PWR-like lumped system rather than a named plant model. A classroom demonstrator of heat removal, boiling/CHF transitions, ECCS, and accident-response trends. | Intended learning role Links trip and decay heat to the continuing need for inventory, core coverage, and heat removal. Shows how pressure governs saturation, discharge, boiling, ECCS availability, and RHR access. Supports scenario demonstrations, intervention exercises, event-timeline review, and CSV analysis. |
| --- | --- |

Teaching position: This module extends point kinetics into conservation-based lumped thermal-hydraulics and qualitative accident progression.

|  | THERMAL-HYDRAULICS AND LOCA SIMULATOR | 03 |
| --- | --- | --- |

2. What it does

| Run major scenarios Normal operation, SBLOCA, LBLOCA, loss of flow, loss of heat sink, and station blackout presets. | Control the plant Adjust rods, reactivity trim, boron, primary flow, steam-generator heat removal, break size, ECCS, AFW, PORV, spray, heaters, and RHR. |
| --- | --- |
| Watch system response Plot neutron power, decay heat, fuel/cladding/coolant temperatures, pressure, inventory, reactivity, and effective flow. | Use automatic systems Enable or disable automatic reactor trip and automatic ECCS logic; observe pressure-dependent injection availability. |
| Follow guided demos Use staged SBLOCA, LBLOCA, loss-of-heat-sink, and station-blackout recovery scripts with explanatory status messages. | Record evidence Freeze or scroll the recorder and log scenario, controls, alarms, state variables, and responses to CSV. |

|  | THERMAL-HYDRAULICS AND LOCA SIMULATOR | 04 |
| --- | --- | --- |

3. What it doesn't do

| Physics not resolved No one- or three-dimensional flow network, component geometry, loop asymmetry, or spatial core model. No two-fluid equations, slip, phase separation, counter-current limits, entrainment, or mechanistic reflood. No validated steam-property package, critical-flow correlation, DNBR method, or mechanistic CHF correlation. No fuel-rod conduction, quench front, cladding oxidation, hydrogen generation, or core-damage progression. | Plant scope not represented No detailed pressurizer levels, steam-generator secondary inventory/pressure, containment, sump, or train logic. No plant-specific pumps, valves, accumulators, protection setpoints, ECCS capacities, or procedures. Not validated as a system or safety-analysis code; thresholds and coefficients are classroom parameters. Warnings, CHF ratio, void, and temperatures illustrate trends and are not acceptance criteria. |
| --- | --- |

Boundary of use: The simulator demonstrates interacting inventories, energies, and protection responses; it does not calculate regulatory margins.

|  | THERMAL-HYDRAULICS AND LOCA SIMULATOR | 05 |
| --- | --- | --- |

4. Physics included

| Power & reactivity Six-group point kinetics with rod, trim, boron, fuel-temperature, and moderator-temperature feedback. | Decay heat Three decay-heat groups initialized at full-power equilibrium and retained after reactor trip. |
| --- | --- |
| Fuel-to-coolant heat transfer Lumped fuel and cladding energy balances with inventory-, coverage-, and flow-dependent conductance. Single-phase, nucleate, transition, and film-boiling regimes with an illustrative CHF ratio. | Coolant mass, energy & pressure A synchronized coolant inventory/stored-energy balance with enthalpy-carrying boundary flows. A saturated pressurizer obtains pressure from the same compact curve used for boiling onset. |
| Inventory & safety injection Pressure-dependent break/PORV flow, evaporation, and staged automatic or manual ECCS injection. Cold injection simultaneously restores mass and changes coolant energy. | Heat sinks & event evidence Steam-generator/AFW removal, spray, heaters, and low-pressure RHR controls. The timeline records trip, ECCS, inventory, boiling regime, CHF, and recovery transitions. |

|  | THERMAL-HYDRAULICS AND LOCA SIMULATOR | 06 |
| --- | --- | --- |

5. Physics simplified

| Single-volume plant model The primary system has one mass inventory, one stored-energy state, one pressure, and one bulk coolant temperature. Fuel and cladding are single thermal nodes; displayed cladding temperature is not a local peak prediction. Core coverage, natural circulation, CHF, void, and boiling regimes use transparent empirical relationships. Break, PORV, evaporation, ECCS, spray, AFW, and RHR remain coefficient-based source/sink terms. | Consistent but approximate thermodynamics A compact saturation table ties pressurizer pressure and boiling onset together; it is not a steam-table package. A fixed latent-heat equivalent represents vaporization energy and flashing discharge. CHF availability scales with pressure, forced flow, and wetted-core fraction; post-CHF conductance degrades smoothly. Bounds and smooth thresholds preserve stable classroom behaviour outside no validated extrapolation range. |
| --- | --- |

|  | THERMAL-HYDRAULICS AND LOCA SIMULATOR | 07 |
| --- | --- | --- |

6. Pedagogical decisions taken

| Slowed kinetics Lambda = 0.055 s is deliberately enlarged so neutron and thermal-hydraulic responses can be followed together. | Controlled substepping Each update is subdivided to model steps no larger than 0.05 s; animation speed does not remove numerical substeps. |
| --- | --- |
| Conservative coupling Mass and stored energy advance with the same effective boundary flows, including cold ECCS and discharge enthalpy. | Visible boiling limits Regime labels, void fraction, and CHF ratio show why heat transfer can improve near boiling and collapse after CHF. |
| Qualitative availability Injection strengthens as pressure falls; RHR becomes useful mainly below about 3.5 MPa; uncovering reduces conductance. | Evidence-rich scenarios Presets, guided recoveries, alarms, transition timeline, strip charts, and CSV logging support prediction-intervention-analysis. |

Current-code clarification: The current model uses a saturation-consistent pressurizer, coolant mass/energy conservation with enthalpy transport, and an illustrative boiling/CHF transition model. These improve internal consistency without making the simulator a best-estimate system code.